# 网络划分

| 部门 | 人数 | ipv4 | vlan | ipv4 网关 | ipv6 | ipv6 网关 |
| --- | --- | --- | --- | --- | --- | --- |
| 市场 | 30 | 192.168.10.1/26 | 10 | 192.168.10.62 | 2001:250:a160:1::1/64 | 2001:250:a160:1::2 |
| 销售 | 13 | 192.168.10.129/28 | 20 | 192.168.10.142 | 2001:250:a160:2::1/64 | 2001:250:a160:2::2 |
| 运维 | 3 | 192.168.10.145/29 | 30 | 192.168.10.150 | 2001:250:a160:3::1/64 | 2001:250:a160:3::2 |
| 开发 | 40 | 192.168.10.65/26 | 40 | 192.168.10.126 | 2001:250:a160:4::1/64 | 2001:250:a160:4::2 |
| DC | / | 210.28.91.1/24 | 50 | 210.28.91.254 | 2001:250:a160:5::1/64 | 2001:250:a160:5::2 |
| 上海 | 20 | 192.168.20.1/27 | 60 | 192.168.20.30 | 2001:250:a160:6::1/64 | 2001:250:a160:6::2 |

| 设备 | 接口 | ipv4 | ipv6 | vlan |
| --- | --- | --- | --- | --- |
| SW1 | e0/0 | 192.168.10.254/30 | 2001:250:a160:7::2/24 | 70 |
| SW1 | e0/1 | 192.168.10.246/30 | / | 100 |
| SW1 | e0/2 | 192.168.10.62/26 | 2001:250:a160:1::2/24 | 10 |
| SW1 | e0/3 | 192.168.10.142/28 | 2001:250:a160:2::2/24 | 20 |
| SW1 | e1/0 | 192.168.10.246/30 | / | 100 |
| SW2 | e0/0 | 192.168.10.249/30 | 2001:250:a160:8::2/24 | 80 |
| SW2 | e0/1 | 192.168.10.245/30 | / | 100 |
| SW2 | e0/2 | 192.168.10.150/29 | 2001:250:a160:3::2/24 | 30 |
| SW2 | e0/3 | 192.168.10.126/26 | 2001:250:a160:4::2/24 | 40 |
| SW2 | e1/0 | 210.28.91.254/24 | 2001:250:a160:5::2/24 | 50 |
| SW2 | e1/1 | 192.168.10.245/30 | / | 100 |
| SW3 | e0/0 | 192.168.20.254/30 | 2001:250:a160:9::2/24 | 90 |
| SW3 | e0/1 | 192.168.20.30/27 | 2001:250:a160:6::2/24 | 60 |
| R1 | e0/0 | 192.168.10.253/30 | 2001:250:a160:7::1/24 | / |
| R1 | e0/1 | 192.168.10.250/30 | 2001:250:a160:8::1/24 | / |
| R1 | e0/2 | 202.226.30.5/30 | / | / |
| R1 | e0/3 | / | 2001:250:a160::2/24 | / |
| R1 | e1/0 | 221.226.30.1/30 | / | / |
| R1 | e1/1 | 172.20.1.3 | / | / |
| R1 | 隧道 | 192.168.10.242/30 | 2001:250:a160:10::1/24 | / |
| R2 | e0/0 | 202.226.30.6/30 | / | / |
| R2 | e0/1 | 221.226.30.9/30 | / | / |
| R2 | e0/2 | 221.226.30.2/30 | / | / |
| R3 | e0/0 | 221.226.30.10/30 | / | / |
| R3 | e0/1 | 192.168.20.253/30 | 2001:250:a160:9::1/24 | / |
| R3 | 隧道 | 192.168.10.241/30 | 2001:250:a160:10::2/24 | / |
| R4 | e0/0 | / | 2001:250:a160::1/24 | / |

# IP 及 VLAN 规划

#PC1

ip 2001:250:a160:1::1/64



#PC2

int e0/0 

no shutdown

ip address 192.168.10.129 255.255.255.240

ipv6 address 2001:250:a160:2::1/64

ip route 0.0.0.0 0.0.0.0 192.168.10.142



#PC3

int e0/0

no shutdown

ip address 192.168.10.145 255.255.255.248

ipv6 address 2001:250:a160:3::1/64

ip route 0.0.0.0 0.0.0.0 192.168.10.150



#PC4

ip 192.168.10.65/26 192.168.10.126

ip 2001:250:a160:4::1/64



#PC5

ip 192.168.20.1/27 192.168.20.30

ip 2001:250:a160:6::1/64



#SERVER

int e0/0

no shutdown

ip address 210.28.91.1 255.255.255.0

ipv6 address 2001:250:a160:5::1/64

ip route 0.0.0.0 0.0.0.0 210.28.91.254



#SW1

vlan 10 # SW1-PC1

int vlan 10

no shutdown

ip address 192.168.10.62 255.255.255.192

ipv6 address 2001:250:a160:1::2/64

int e0/2

no shutdown

sw ac vlan 10

vlan 20 # SW1-PC2

int vlan 20

no shutdown

ip address 192.168.10.142 255.255.255.240

ipv6 address 2001:250:a160:2::2/64

int e0/3

no shutdown

sw ac vlan 20

vlan 70 # SW1-R1

int vlan 70

no shutdown

ip address 192.168.10.254 255.255.255.252

ipv6 address 2001:250:a160:7::2/64

int e0/0

no shutdown

sw ac vlan 70



#SW2

vlan 30 # SW2-PC3

int vlan 30

no shutdown

ip address 192.168.10.150 255.255.255.248

ipv6 address 2001:250:a160:3::2/64

int e0/2

no shutdown

sw ac vlan 30

vlan 40 # SW2-PC4

int vlan 40

int vlan 40

no shutdown

ip address 192.168.10.126 255.255.255.192

ipv6 address 2001:250:a160:4::2/64

int e0/3

no shutdown

sw ac vlan 40

vlan 50 # SW2-SERVER

int vlan 50

no shutdown

ip address 210.28.91.254 255.255.255.0

ipv6 address 2001:250:a160:5::2/64

int e1/0

no shutdown

sw ac vlan 50

vlan 80 # SW2-R1

int vlan 80

no shutdown

ip address 192.168.10.249 255.255.255.252

ipv6 address 2001:250:a160:8::2/64

int e0/0

no shutdown

sw ac vlan 80



#SW3

vlan 60 # SW3-PC5

int vlan 60

no shutdown

ip address 192.168.20.30 255.255.255.224

ipv6 address 2001:250:a160:6::2/64

int e0/1

no shutdown

sw ac vlan 60

vlan 90 # SW3-R3

int vlan 90

no shutdown

ip address 192.168.20.254 255.255.255.252

ipv6 address 2001:250:a160:9::2/64

int e0/0

no shutdown

sw ac vlan 90



#R1

int e0/0 # R1-SW1

no shutdown

ip address 192.168.10.253 255.255.255.252

ipv6 address 2001:250:a160:7::1/64

int e0/1 # R1-SW2

no shutdown

ip address 192.168.10.250 255.255.255.252

ipv6 address 2001:250:a160:8::1/64

int e0/2 # R1-R2

no shutdown                              

ip address 202.226.30.5 255.255.255.252  

int e1/0 # R1-R2

no shutdown                            

ip address 221.226.30.1 255.255.255.252

int e0/3 # R1-R4

no shutdown

ipv6 address 2001:250:a160::2/64



#R2

int loopback0 # R2-lo0

no shutdown

ip address 8.8.8.8 255.255.255.255

int e0/0 # R2-R1

no shutdown

ip address 202.226.30.6 255.255.255.252

int e0/1 # R2-R3

no shutdown

ip address 221.226.30.9 255.255.255.252

int e0/2 # R2-R1

no shutdown                            

ip address 221.226.30.2 255.255.255.252



#R3

int e0/0 # R3-R2

no shutdown

ip address 221.226.30.10 255.255.255.252

int e0/1 # R3-SW3

no shutdown

ip address 192.168.20.253 255.255.255.252

ipv6 address 2001:250:a160:9::1/64



#R4

int e0/0 # R4-R1

no shutdown

ipv6 address 2001:250:a160::1/64

# DHCP 配置

#SW1

service dhcp                                

ip dhcp excluded-address 192.168.10.62

ip dhcp pool dhcp-pool1

network 192.168.10.0 255.255.255.192

dns-server 114.114.114.114

default-router 192.168.10.62

lease 1



#PC1

ip dhcp

SW1

# 远程配置管理

#R1

username se secret lab@seu

line vty 0 4

login local

transport input telnet



#SW1

username se secret lab@seu

line vty 0 4

login local

transport input telnet

access-list 150 deny ip 0.0.0.0 255.255.255.255 192.168.10.0 0.0.0.255 sq telnet

access-list 150 permit ip any any

int vlan 10

ip access-group 150 in

int vlan 20

ip access-group 150 in



#SW2

username se secret lab@seu

line vty 0 4

login local

transport input telnet

access-list 150 deny ip 0.0.0.0 255.255.255.255 192.168.10.0 0.0.0.255 sq telnet

access-list 150 permit ip any any

int vlan 40

ip access-group 150 in

int vlan 50

ip access-group 150 in

SW1

SW2

PC1

PC3

SERVER

# 防止环路

#SW1

int range e0/1-1,e1/0-0

channel-group 1 mode active

vlan 100

int vlan 100

no shutdown

ip address 192.168.10.246 255.255.255.252

int range e0/1-1,e1/0-0

sw ac vlan 100



#SW2

int range e1/1-1,e0/1-1

channel-group 1 mode passive

vlan 100

int vlan 100

no shutdown

ip address 192.168.10.245 255.255.255.252

int range e1/1-1,e0/1-1

sw ac vlan 100

SW1

SW2

# 路由配置

#R1

router ospf 100

router-id 1.1.1.1

redistribute bgp 65511 subnets

network 192.168.10.252 0.0.0.3 area 0

network 192.168.10.248 0.0.0.3 area 0

default-information originate always

router bgp 65511

no bgp log-neighbor-changes

neighbor 221.226.30.2 remote-as 65512

neighbor 202.226.30.6 remote-as 65512

router bgp 65511

redistribute ospf 100

ipv6 route 2001:250:a160:1::/64 2001:250:a160:7::2

ipv6 route 2001:250:a160:2::/64 2001:250:a160:7::2

ipv6 route 2001:250:a160:3::/64 2001:250:a160:8::2

ipv6 route 2001:250:a160:4::/64 2001:250:a160:8::2

ipv6 route 2001:250:a160:5::/64 2001:250:a160:8::2

ipv6 route ::/0 2001:250:a160:10::2

ipv6 unicast-routing



#R2

router bgp 65512

no bgp log-neighbor-changes

neighbor 221.226.30.1 remote-as 65511

neighbor 202.226.30.5 remote-as 65511

neighbor 221.226.30.10 remote-as 65512

neighbor 221.226.30.10 next-hop-self  

router bgp 65512

redistribute connected

redistribute static

ip route 192.168.20.0 255.255.255.0 221.226.30.10



#R3

ip route 192.168.10.0 255.255.255.0 221.226.30.9

ip route 192.168.30.0 255.255.255.0 192.168.30.254

ip route 0.0.0.0 0.0.0.0 221.226.30.9

ipv6 route ::/0 2001:250:a160:10::1

ipv6 route 2001:250:a160:6::/0 2001:250:a160:9::2

ipv6 unicast-routing



#R4

ipv6 route ::/0 2001:250:a160::2



#SW1

router ospf 100

router-id 2.2.2.2

network 192.168.10.244 0.0.0.3 area 0

network 192.168.10.252 0.0.0.3 area 0

network 192.168.10.0 0.0.0.63 area 0

network 192.168.10.128 0.0.0.15 area 0

int vlan 70

ip ospf priority 0

int vlan 100

ip ospf priority 0

ipv6 route ::/0 2001:250:a160:7::1

ipv6 unicast-routing



#SW2

router ospf 100

router-id 3.3.3.3

network 192.168.10.244 0.0.0.3 area 0

network 192.168.10.248 0.0.0.3 area 0

network 192.168.10.64 0.0.0.63 area 0

network 192.168.10.144 0.0.0.63 area 0

network 210.28.91.0 0.0.0.255 area 0

int vlan 80

ip ospf priority 0

int vlan 100

ip ospf priority 0

ipv6 route ::/0 2001:250:a160:8::1

ipv6 unicast-routing



#SW3

ip route 0.0.0.0 0.0.0.0 192.168.20.253

ipv6 route ::/0 2001:250:a160:9::1

ipv6 unicast-routing

R1

SW1

SW2

R1

R2

# 隧道配置

#R1

int tunnel1

no shutdown

ipv6 enable

ipv6 address 2001:250:a160:10::1/64

tunnel source 221.226.30.1

tunnel destination 221.226.30.10

tunnel mode ipv6ip



#R3

int tunnel1  

no shutdown

ipv6 enable

ipv6 address 2001:250:a160:10::2/64

tunnel source 221.226.30.10

tunnel destination 221.226.30.1

tunnel mode ipv6ip

# 策略配置

#SW1

access-list 100 permit ip 192.168.10.0 0.0.0.63 192.168.10.144 0.0.0.7 

access-list 100 permit ip 192.168.10.0 0.0.0.63 192.168.10.64 0.0.0.63       

access-list 100 permit ip 192.168.10.128 0.0.0.15 192.168.10.144 0.0.0.7

access-list 100 permit ip 192.168.10.128 0.0.0.15 192.168.10.64 0.0.0.63

route-map out permit 10

match ip address 100

set ip next-hop 192.168.10.253

ip local policy route-map out



#SW2

access-list 100 permit ip 192.168.10.144 0.0.0.7 192.168.10.0 0.0.0.63 

access-list 100 permit ip 192.168.10.144 0.0.0.7 192.168.10.128 0.0.0.15 

access-list 100 permit ip 192.168.10.64 0.0.0.63 192.168.10.0 0.0.0.63       

access-list 100 permit ip 192.168.10.64 0.0.0.63 192.168.10.128 0.0.0.15 

route-map out permit 10 

match ip address 100 

set ip next-hop 192.168.10.250

ip local policy route-map out



#R1

access-list 1 permit 192.168.10.0 0.0.0.255 

access-list 2 permit 210.28.91.0 0.0.0.255 

access-list 99 permit any                   

route-map bgp1 permit 10   #互联专线

match ip address 1

set metric 1000

route-map bgp1 permit 20

match ip address 2  

set metric 2000  

route-map bgp1 permit 30

match ip address 99 

router bgp 65511

neighbor 221.226.30.2 route-map bgp1 out 

route-map bgp2 permit 10  #DC专线

match ip address 1

set metric 2000

route-map bgp2 permit 20

match ip address 2  

set metric 1000  

match ip address 99

router bgp 65511

neighbor 202.226.30.6 route-map bgp2 out



#R2

access-list 110 permit ip 192.168.10.0 0.0.0.255 8.8.8.8 0.0.0.0

access-list 120 permit ip 210.28.91.0 0.0.0.255 8.8.8.8 0.0.0.0

access-list 99 permit any                   

route-map bgp1 permit 10  #互联专线

match ip address 110

set metric 1000

route-map bgp1 permit 20

match ip address 120  

set metric 2000  

route-map bgp1 permit 30

match ip address 99              

router bgp 65512

neighbor 221.226.30.1 route-map bgp1 out 

route-map bgp2 permit 10  #DC专线

match ip address 110

set metric 2000

route-map bgp2 permit 20

match ip address 120  

set metric 1000  

route-map bgp2 permit 30

match ip address 99              

router bgp 65512

neighbor 202.226.30.5 route-map bgp2 out

PC1->PC4

R2

R1

# 安全机制

#SW2

int e0/3

sw port-security mac-address 0050.7966.680c

sw port-security maximum 1 

sw port-security violation shutdown

sw mode access                     

sw port-security



#R1

access-list 101 deny icmp 192.168.10.0 0.0.0.63 210.28.91.0 0.0.0.255 

access-list 101 deny icmp 192.168.10.128 0.0.0.15 210.28.91.0 0.0.0.255      

access-list 101 permit ip any any

int e0/0

ip access-group 101 in



#R1

crypto isakmp policy 2

hash md5

authentication pre-share

group 2

crypto isakmp key lab@seu address 221.226.30.10

crypto ipsec transform-set tor3 esp-des esp-md5-hmac

mode transport

crypto ipsec profile ipsec

set transform-set tor3

int tunnel0

tunnel mode ipip

ip address 192.168.10.242 255.255.255.252

tunnel protection ipsec profile ipsec

ip route 192.168.20.0 255.255.255.0 192.168.10.241



#R3

crypto isakmp policy 2

hash md5

authentication pre-share

group 2

crypto isakmp key lab6 address 221.226.30.1

crypto ipsec transform-set tor1 esp-des esp-md5-hmac

mode transport

crypto ipsec profile ipsec

set transform-set tor1

int tunnel0

tunnel mode ipip

ip address 192.168.10.241 255.255.255.252

tunnel protection ipsec profile ipsec

ip route 210.28.91.0 255.255.255.0 192.168.10.242

SW2

PC1

PC2

R3

# NAT 配置

#R1

int e1/1

no shutdown

ip address 172.20.1.3 255.255.255.0

ip route 0.0.0.0 0.0.0.0 172.20.1.254 

router bgp 65511

no bgp log-neighbor-changes

neighbor 172.20.1.254 remote-as 65513	

access-list 5 permit 192.168.10.0 0.0.0.255

access-list 5 permit 192.168.20.0 0.0.0.255

ip nat inside source list 5 int e1/1 overload

int e0/0

ip nat inside

int e0/1

ip nat inside 

int e0/2

ip nat inside

int e1/0

ip nat inside

int e1/1

ip nat outside

R1